DOC NO:  EXOS-SAF-002-0

DATE:  09/21/2026

STATUS:  DRAFT

Carbon Fiber / Composite Machining Dust Control

Equipment / System No ___________________     Date _____________     Performed by ______________________________

SAFETY / ENVIRONMENTAL

Verify that all personnel who will be involved with this activity are familiar with this procedure, any risks or hazards that may be encountered, possess the required safety equipment, and are familiar with the site safety and environmental policies.

OBJECTIVE

The purpose of this procedure is to protect Exos personnel from the respiratory and contact health hazards of carbon-fiber / composite machining dust by requiring source capture at the tool, true-HEPA extraction, disposable sealed-bag waste handling, and P100 respiratory protection, while carving combustible-metal fines out of scope.

OVERVIEW

Machining cured CFRP releases respirable carbon fibers and resin dust that are an inhalation and skin/eye irritant; the fibers are also electrically conductive (they short electronics and foul motors) and the resin fraction is combustible. This procedure establishes the standard control method: capture dust at the point of cut with a tool-mounted shroud tied to a true-HEPA (99.97% at 0.3 micron) extractor fitted with a cyclone pre-separator, collect into DISPOSABLE self-sealing filter bags that are sealed and discarded without ever handling the dust, and back that up with P100 respirators and wet/HEPA housekeeping. It applies to composites only. Cutting aluminum, titanium, or magnesium generates combustible metal fines that must NOT be dry-vacuumed with this equipment; those operations require a separate wet-collection method and are excluded here.

PROCEDURE

1 - Scope and applicability

☐  Applies to all cutting, drilling, milling, sanding, grinding, and trimming of cured carbon-fiber-reinforced polymer (CFRP) and comparable composite laminates at Exos facilities.

☐  COMPOSITES ONLY. This procedure and its dry-HEPA equipment are NOT approved for aluminum, titanium, or magnesium fines - dry vacuuming combustible metal dust is a fire/deflagration hazard. Metal machining requires wet collection under a separate procedure.

☐  Do not co-mingle metal chips/fines with composite dust in the same extractor or waste bag.

☐  If an operation produces both composite and metal dust, stop and route to EHS for a control determination before proceeding.

2 - Hazard summary

☐  Respiratory: respirable carbon fibers and resin dust are an inhalation irritant; some resin systems are sensitizers. Treat as a health-hazard dust requiring HEPA capture and respiratory protection.

☐  Contact: fibers cause skin and eye irritation; cover skin and protect eyes.

☐  Electrical: carbon fiber dust is conductive - it shorts electronics and can foul/arc a vacuum motor; keep it captured and contained.

☐  Fire/combustible: the resin fraction is combustible. This SOP controls the HEALTH hazard; ignition risk is managed by housekeeping (Task 6) and the Dust Hazard Analysis (Task 7). The standard Festool extractor is HEPA-rated but NOT NFPA/ATEX combustible-dust rated - it is acceptable only while the DHA supports it.

3 - Required equipment

☐  Extractor: Festool CT 36 EI AC HEPA (P/N 577874) or equivalent true-HEPA (99.97% at 0.3 micron) extractor with automatic filter cleaning (AUTOCLEAN). AUTOCLEAN is required - composite dust blinds filters quickly.

☐  Spare HEPA main filter: Festool HEPA-HF-CT 26/36/48 PTFE (P/N 205412).

☐  Filter bags: DISPOSABLE self-sealing fleece bags only - Festool SC-FIS-CT 36/5 (confirm CT 36 EI-matched SKU). NEVER use the reusable Longlife fabric bag for composite dust.

☐  Cyclone pre-separator: Festool CT-VA-20 (P/N 204083) or Oneida Dust Deputy - drops the bulk of the dust before the HEPA and preserves airflow/filter life.

☐  Source-capture shroud/dust boot matched to the tool in use, tied to the extractor via the antistatic hose.

☐  PPE: NIOSH P100 respirator (e.g., 3M 6300 + 2091 filters), disposable coveralls (Tyvek), nitrile gloves, and sealed eye protection.

4 - Setup and operation

☐  Confirm the HEPA filter and a fresh disposable SC-FIS-CT bag are installed and the cyclone pre-separator is inline.

☐  Attach the tool-mounted shroud/boot and the antistatic hose; verify capture is at the point of cut, not room air.

☐  Enable AUTOCLEAN; where a Festool cordless tool is used, pair Bluetooth auto-start so the extractor runs whenever the tool runs.

☐  Don P100 respirator (fit-tested), coveralls, gloves, and eye protection BEFORE starting.

☐  Run the extractor for several seconds before and after each cut to clear the capture zone.

☐  For high-volume or enclosed work, add negative-pressure containment (ZipWall + poly + a HEPA negative-air scrubber) as directed by EHS.

☐  Do not operate if the shroud is missing, the hose/filter is damaged, or the bag is full.

5 - Filter bag and waste handling

☐  Use DISPOSABLE self-sealing bags only. When a bag is full (or at end of job for hazardous dust), seal the bag collar WITHOUT emptying or handling the dust, and remove it.

☐  Do NOT empty and reuse any bag that has held composite dust; do not use the reusable Longlife bag for this material.

☐  Place the sealed bag in a labeled waste container. Wear P100 + gloves during bag change.

☐  Disposal classification: cured CFRP dust is generally industrial/special solid waste, NOT RCRA hazardous - BUT verify against the resin-system SDS and confirm the route with the waste hauler / EHS before setting a standing practice. Uncured/partially-cured resin or additives can change the classification.

☐  HEPA main filter: not a per-use consumable. Replace on a periodic/condition basis; when removed, bag and dispose it as contaminated dust the same way.

☐  Never dry-sweep, blow down with compressed air, or vacuum composite dust with a non-HEPA shop vac.

6 - Housekeeping and cleanup

☐  Clean surfaces by wet-wipe or HEPA vacuum only - never dry-sweep or use compressed air (both re-aerosolize fibers).

☐  Keep horizontal surfaces, ledges, and equipment free of accumulated dust - accumulation is the combustible-dust risk driver.

☐  Wipe down the extractor exterior, hose, and shroud after use.

☐  Remove and bag disposable coveralls and gloves at the work area; wash hands and any exposed skin.

7 - Combustible-dust governance

☐  A Dust Hazard Analysis (DHA) per NFPA 652 shall be performed for the composite machining operation to determine whether the actual dust and quantities cross the combustible-dust threshold.

☐  If the DHA determines a rated system is required, step up to an NFPA/ATEX-rated conductive HEPA extractor (e.g., Nilfisk VHC / Fein antistatic H-class). This SOP's standard Festool extractor is approved only while the DHA supports it.

☐  Maintain no-accumulation housekeeping and control ignition sources in the machining area regardless of the equipment tier.

☐  Re-run or review the DHA after any material change in resin system, throughput, or equipment.

8 - Training and records

☐  Personnel performing composite machining shall be trained on this procedure, respirator fit-tested (P100), and briefed on the metals carve-out before independent work.

☐  Record training and fit-test completion per the HR/training process.

☐  Log the DHA and any exposure/industrial-hygiene assessments as controlled records.

☐  Review this procedure annually or after any incident, DHA change, or equipment change.

Completion

☐  Record completion of the activity in the Exos Work Management System (Mission Control).

Owner: Machine Shop Lead / Exo (CEO approves)   ·   Reference documents: NFPA 652 (Standard on the Fundamentals of Combustible Dust), NFPA 654, OSHA 29 CFR 1910.1000 (air contaminants) & 1910.134 (respiratory protection), resin-system SDS, EXOS-SAF-001-0 (Cryogenic LOX Handling), EXOS-MFG-001-0 (NLX2500 Tool-Post Calibration)