MEETING NOTES — NASA MSFC RDRE TECHNICAL DISCUSSION

October 6, 2026  |  1:00 PM CDT  |  Virtual

⚠ INTERNAL ONLY — For Exos internal use only. Do not distribute outside Exos without Jon Quinn approval.

1. Attendees

NASA MSFC

•  Tom Teasley — RDRE Technical Lead, INROADS program, ER13 (combustion devices)

•  Seth Bell — Industry & Commercial Partnerships Manager (also covers OGA / Other Government Agencies)

•  Mike — Partnerships (surname unknown; Seth's colleague in the partnerships office)

•  Chris Hill — Project Manager / Approver for INROADS (not on the call; must approve any SAA)

Exos Aerospace

•  Jon Quinn — Cofounder & CEO

•  Phil Gard — Chief Engineer (joined call late, via phone)

2. Background & Purpose

This was the first substantive technical-plus-business meeting between Exos Aerospace and NASA MSFC's RDRE team. Exos is conducting a trade-study down-select between (a) licensing the NASA MSFC INROADS RDRE design and (b) the GTL fully-machined RDRE path, for potential integration on a BLK3-class vehicle flight demonstration. This meeting opened both the technical and agreement-structure discussions.

Transcription note: 'INROADS' (or 'in roads') appears to be the NASA MSFC RDRE development program name. '9,000–12,000 kilohertz' is almost certainly 9–12 kHz (kilohertz) not megahertz.

3. Technical Facts (per Teasley, verbal — 2026-10-06)

3.1 Engine Performance & Tested Hardware

•  Maximum tested thrust: 5,800 lbf (sea level, pressure-fed to date).  (per Teasley, verbal)

•  Baseline design: ~20 lb/s mass flow → ~5,000–6,000 lbf at sea level (sea-level atmosphere).  (per Teasley, verbal)

•    At altitude (e.g., 10,000 ft) same conditions yield ~8,000 lbf+.  (per Teasley, verbal)

•  Baseline operating conditions: ~1,000 psi chamber pressure / ~1,700 psi inlet pressure.  (per Teasley, verbal)

•  Throttle capability: baselined at 50%; possibly 40%, unconfirmed — thrust chamber only, other components may limit further.  (per Teasley, verbal)

•  Cooling: all regenerative — no film cooling used.  (per Teasley, verbal)

•  A notional 10,000-lbf engine system is baselined for a turbo-pump-fed methane-rich gas generator cycle (thrust chamber same design, 50–100% = ~5,000–10,000 lbf).  (per Teasley, verbal)

3.2 Efficiency

•  ≥98% combustion efficiency at ≥700 psi chamber pressure.  (per Teasley, verbal)

•  ≥90% combustion efficiency above ~300 psi chamber pressure.  (per Teasley, verbal)

•  ISP ~250 s at ~300 psi Pc / 20 lb/s / ~5,000 lbf (LOX-methane or LOX-ethanol close to same).  (per Teasley, verbal)

3.3 Loads Environment

•  Measured loads: up to ~1,000+ g in some test configurations.  (per Teasley, verbal)

•  Dominant frequency range: ~9–12 kHz.  (per Teasley, verbal)

Transcription artifact: transcript reads '9,000–12,000 kilohertz' which is almost certainly 9–12 kHz.

•  Loads environment data book: not yet released — team is still correlating scattershot data from different test-article locations; data not yet interpretable enough to publish.  (per Teasley, verbal)

•  Structural damage observed: early testing saw metal C-rings shattering; pushing operating frequency higher shifted to smaller instrumentation ports failing.  (per Teasley, verbal)

•  No wall erosion of the combustion chamber surface has been observed in any MSFC test.  (per Teasley, verbal)

•  Detonations appear localized within the first ~0.25 inch from the injector face.  (per Teasley, verbal)

•  Resonator cavity testing showed it did mitigate targeted frequencies.  (per Teasley, verbal)

3.4 Inlet Pressure / Exos Interface Compatibility

•  750 psi inlet would require a redesign and remanufacture of the outer body to open the throat area (reduce chamber pressure and inlet pressure to match Exos supply capability).  (per Teasley, verbal)

•  Running at ~25 lb/s for ~6,000 lbf SL requires ~1,200 psi inlet — pushes toward pump augmentation.  (per Teasley, verbal)

•  Options for Exos tank pressure: 750 / 1,000 / 1,200 psi — Exos can manufacture CF tanks to any of these ratings.  (discussed on call)

•  MSFC did look for LOX/CH4 turbo-pump designs below 10,000 lbf in the NASA repository — none found.  (per Teasley, verbal)

•  Alternative noted: Ursa Major Hadley engine (~5,000 lbf, LOX/kerosene, oxidizer-rich) — turbo-pump could potentially be adapted with an RDRE thrust chamber.  (per Teasley, verbal)

3.5 Hardware Availability & Timeline

•  Existing non-flight-weight hardware: available now; no performance guarantee, no duration guarantee.  (per Teasley, verbal)

•  Higher-confidence hardware: post-1B test series (≈ March–May 2027) and post-1C series — likely available end-2027 at earliest.  (per Teasley, verbal)

•  New thrust chamber (license & adapt current design): ~12–18 months to do correctly.  (per Teasley, verbal)

•  Exos-owned design with Teasley coaching: approximately 2 years.  (per Teasley, verbal)

•  Turbomachinery: several years; even the INROADS program was baselined 4–5 years and it was considered aggressive.  (per Teasley, verbal)

•  Manufacturing steps (additive path): ~10 days print + ~3 weeks depowder + optional CT scan + HIP up to ~2 weeks + flow test up to ~2 weeks.  (per Teasley, verbal)

•  GRCop powder supply easing: primary suppliers bringing additional nozzles online; several large shipments reportedly landed ~2 months ago.  (per Teasley, verbal)

•  Fastest path overall: license the current INROADS design (adapts to Exos operating conditions, not flight-weight but extensible).  (per Teasley, verbal)

3.6 Vacuum / In-Space Operation

•  Vacuum restart is possible — some MSFC work done, details not releasable.  (per Teasley, verbal)

•  No flight demonstration is funded or planned in the current INROADS program; any flight demo is 5–10+ years out under NASA's current program.  (per Teasley, verbal)

•  Ground-based vacuum chamber demo discussion exists, but is not funded.  (per Teasley, verbal)

Jon commented Exos could potentially do a vacuum/in-space flight demo in 2 years or less — noted as an Exos aspiration, not a NASA commitment.

3.7 INROADS Program Papers / IP

•  Heat-transfer correlation paper in journal review (JNF / JANNAF); single equation predicts convective heat transfer coefficient for LOX/CH4 RDE hot wall.  (per Teasley, verbal)

•  Paper in Journal of Propulsion & Power: design and prediction of wave count and wave speed for any given geometry — validated against ~300 hot fires.  (per Teasley, verbal)

4. Proposed Agreement Path (per MSFC — Teasley / Mike / Seth, verbal)

•  Step 1: Exos provides firm written requirements document — what they want, operating conditions, expectations, schedule/budget context.

•  Step 2: Teasley socializes requirements with PM Chris Hill. Hill must approve before any agreement is executed.

•  Step 3: Umbrella Reimbursable Space Act Agreement (SAA), structured with annexes allowing phased scope additions.

•    Annex 1 starts with paid consultation — Exos pays for Teasley's time; Seth and Mike are overhead / no direct cost.  (per Mike/Seth, verbal)

•    Subsequent annexes added as scope / needs evolve.  (per Seth, verbal)

•  Licensing path: Exos has NOT yet engaged NASA Tech Transfer. Teasley confirmed this is the separate, necessary step for an IP license.

All Space Act Agreements are public documents — searchable on NASA.gov. Exos has 1–2 existing SAAs.

5. Exos-Stated Intents (on call — Jon / Phil)

The following reflects what Jon and Phil stated on the call as Exos's intent/goals. These are pre-decisional and subject to board approval and trade-study completion.

•  Flight objective: hover, then fly — generate in-flight data on isolation/damping requirements and in-flight RDRE behavior not available from ground testing.

•  Target burn duration: ~85 seconds.

•  Propellant: LOX/CH4 or LOX/ethanol (tank sizing is ~1:1 between the two).

•  Thrust target: ~6,000 lbf sea level.

•  Inlet pressure options Exos can supply: 750 / 1,000 / 1,200 psi (CF tank manufacturing in-house).

•  Approximate program budget mentioned: ~$10M for a BLK3-class flight demo.

•  Noted existing Exos advantage: previously flew a NASA active vibration damper — aware of what an isolator/damper looks like.

⚠ INTERNAL ONLY — Do NOT describe Exos as an 'RDRE propulsion provider' or as offering propulsion packages. RDRE remains a trade-study down-select (MSFC license vs. GTL machined). Exos does not own the MSFC RDRE technology.

6. Open Questions

•  Exact Exos inlet pressure requirement (750 / 1,000 / 1,200 psi) — drives whether outer-body redesign is needed.

•  Propellant selection: LOX/CH4 vs. LOX/ethanol — to be confirmed in requirements.

•  Mass flow and thrust precision: Phil to confirm numbers vs. Teasley's 20 lb/s baseline.

•  Is ~6,000 lbf achievable at 750 psi inlet without outer-body redesign? — Teasley to confirm after seeing firm requirements.

•  Hadley engine turbo-pump adaptation: path/cost/timeline vs. clean-sheet electric pump at 5,000–6,000 lbf?

•  'RCG' data — confirm exact meaning per Dankanich context.

•  Has Exos had contact with NASA Tech Transfer? (Teasley confirmed they have not — needs to be initiated for licensing path.)

7. Actions

A-1: Jon / Phil — Draft and finalize firm written RDRE requirements document; send to Teasley for Chris Hill review. [OWNER: Jon — Decision posted 2026-10-07]

A-2: Phil — Confirm precise mass flow, inlet pressure envelope, and thrust requirement; validate against Teasley's 20 lb/s baseline. [OWNER: Phil]

A-3: Jon — Initiate contact with NASA Tech Transfer office re: INROADS license path (separate from SAA / Teasley). [OWNER: Jon]

A-4: Jon — Loop in Dankanich on 10/6 meeting outcome; update on GRCop supply and PSI connection (pending). [OWNER: Jon]

A-5: Seth Bell (MSFC) — Prepare umbrella SAA framework for when requirements are ready; Exos to confirm schedule/timeline. [OWNER: Seth]

Exos Aerospace Systems & Technologies, Inc.  |  299 FM 1903, Greenville, TX 75402  |  jquinn@exosaero.com

INTERNAL — Do not distribute. Proprietary — Unclassified.