DOC NO:  EXOS-ITZ-002-0

DATE:  08/21/2026

STATUS:  DRAFT

ITZ Container Security Monitoring System

Equipment / System No ___________________     Date _____________     Performed by ______________________________

SAFETY / ENVIRONMENTAL

Verify that all personnel who will be involved with this activity are familiar with this procedure, any risks or hazards that may be encountered, possess the required safety equipment, and are familiar with the site safety and environmental policies.

OBJECTIVE

The purpose of this procedure is to define the operation, monitoring, maintenance, and response for the ITZ container's physical and electronic security monitoring system, ensuring tamper-evident, access-controlled custody of international payloads.

OVERVIEW

This procedure frames how the GSS Global Secure ITZ container is secured and monitored end to end. The system must operate self-powered (battery/solar, cellular uplink) because the pad has no facility power. Detailed configuration is supplied by Security (Steve Norsworthy); operators are trained on the security software by Global Secure Shipping. Access follows a tiered authorization model under the ITZ Technology Control Plan.

PROCEDURE

Task 1 - System description and components (self-powered)

☐  [Steve to detail] Document monitoring components: six-sided intrusion/hole detection, door-opening detection, CCTV, access control, environmental sensors, and two-way/cellular communications.

☐  DESIGN CONSTRAINT: system is SELF-POWERED (battery and/or solar) with a defined battery-backup runtime and cellular uplink - no facility power at the pad.

☐  Record system make/model, software, and version.

Task 2 - Access control and authorization (tiered, per Technology Control Plan)

☐  Maintain the authorized-access list under the ITZ Technology Control Plan (TCP), applying least-privilege and logging every entry/exit (who, when, purpose).

☐  Tier 1 - Exos operators: full container operations and payload handling; U.S. persons, trained, under the TCP.

☐  Tier 2 - Host partner (Edgar Johansson + designated Montana Spaceport team): trained on access operation (Exos-supported; GSS security-software training), facility/container access; handling of controlled payloads only if U.S. persons under NDA + TCP, otherwise host/logistics support only.

☐  Tier 3 - Emergency/security response (Butte airport security forces / local law enforcement): physical / break-glass emergency access only, logged - no payload handling.

☐  [Steve to detail] Credential issuance/revocation; control foreign-person access per export-control (deemed-export) requirements.

Task 3 - Monitoring and alarm response

☐  [Steve to detail] Define normal monitoring, alerting thresholds, and the monitoring station/uplink.

☐  [Steve to detail] Define alarm response and escalation (who is notified, response times, law-enforcement/Airport coordination).

☐  Record all alarms and dispositions.

Task 4 - Tamper-evidence, seals, and chain-of-custody

☐  Apply and record numbered tamper-evident seals on the container and payload enclosures.

☐  Verify seal integrity at each custody transfer; document breaks and causes.

☐  Maintain the chain-of-custody log consistent with EXOS-ITZ-003.

Task 5 - Logging, review, and audit

☐  Retain access, alarm, and video logs per the records schedule.

☐  Perform periodic review/audit of logs and system health.

☐  Report anomalies to the Security owner.

Task 6 - Training and maintenance

☐  Complete security software training delivered by Global Secure Shipping before operating the system; retain training records.

☐  [Steve to detail] Preventive maintenance, calibration, and test schedule for sensors/CCTV.

☐  Verify the system after any container move or power event (see EXOS-ITZ-001).

Completion

☐  Record completion of the activity in the Exos Work Management System (Mission Control).

Owner: Steve Norsworthy / Security   ·   Reference documents: DHS CRADA [ref]; Butte ITZ MOA; EXOS-ITZ-001; EXOS-ITZ-003; ITZ Technology Control Plan (TCP); ITAR 22 CFR 120-130; EAR 15 CFR 730-774; CMMC 2.0 / NIST SP 800-171 (as applicable); Global Secure Shipping (GSS) training materials.